# Section 15

##### LASER SAFETY TIPS

________________________________________________________________

- Locate laser beam at waist level or below.  Do not place the beam at eye level.

- Close and cover your eyes when stooping down around the laser beam (where you will pass the beam at eye level).

- When leaning over the table, beware of the laser beam directed upward.

- Enclose as much of the beam as possible.

- Do not direct laser beam toward doors or windows.

- Terminate or reflect the laser beam with fire-resistant beam stops. Anodized aluminum painted black (which may not be necessarily fire-resistant) can work well for this purpose.

- Use surfaces that minimize specular reflections.

- Implement effective controls so that the Laser Operator is not exposed to laser intra beam hazards.

- Make sure warning/indicator lights can be seen through protective filters.

- If you can see the beam through your laser eyewear, you are not fully protected.

- View applications remotely.

- Do not wear watches or reflective jewelry around Class 3B or Class 4 lasers.

- In reality, all interlocks are defeatable.

- The best defense is good understanding of the intra beam laser hazards.

- For alignment:

  - Isolate Process.

  - Use lowest practical power.

  - View diffuse reflections only.

  - Use IR/UV viewing cards/eyewear.

  - Where possible, use low power HeNe laser.